| 1.0 | Technical Specifications: General | 849 |
| --- | --- | --- |
| 1.1 | Brand new and latest model | Yes |
| 1.2 | Capable of reaping, threshing and binding operations into a single operation | Reaping and Threshing |
| 1.3 | Capable of harvesting maize, wheat, sunflower, and soybean crops | Yes |
| 1.4 | Supplied with appropriate heads for different crops | Yes |
| 1.5 | Threshing technology: Drum threshing | Yes |
| 1.6 | Wheelbase: 2.0 – 2.5 m | 2.9m |
| 1.7 | Dimension, without Header, Length x Width x Height (mm): 10,760 x 3,200 x 3,400 | 6300x2400x3330 |
| 1.8 | Steering: Hydrostatic or hydraulic | Yes |
| 1.9 | Fitted with a foot pedal stopping the combine and resuming at the same speed | No |

| TECHNICAL SPECIFICATIONS REQUIRED |  |  | TECHNICAL SPECIFICATIONS OFFERED |
| --- | --- | --- | --- |
|  |  |  | PLEASE FILL OUT ALL THE SPACES BELOW TO SPECIFY THE EQUIPMENT BEING OFFERED FOR THE OFFER TO BE VALID |
| 1.10 | Cabin type: Ergonomically integrated. Protected from vibration. Equipped with rear-view mirrors, canopy, front and backlights for night work. |  |  |
| 1.11 | Fuel tank capacity: 120 to 160 litres |  | 175 litres |
| 1.12 | Grain tank: 1 – 3 m3 |  | 1.7 m3 |
| 1.13 | Weight without header: 3200 – 3800 kg |  | 344 5 |
| 1.14 | Noise Level, db(A), maximum: 95 |  | 93.2 dB (A) |
| 2.0 | Technical Specifications: Engine |  |  |
| 2.1 | Type: Diesel-powered engine |  |  |
| 2.2 | Engine power: 50-60 HP (80 – 85 kW) |  | 54.4 HP |
| 2.3 | Rating: 2200 - 2600 rpm |  | 2200 ± 50 RPM |
| 2.4 | Water cooling |  | Yes |
| 2.5 | Direct injection |  | Yes |
| 2.6 | The engine is capable of operating in the summer season of arid and semi-arid climates with temperatures ranging from 30° to 45°C, without overheating |  | Yes |
| 3.0 | Technical Specifications: Transmission System |  |  |
| 3.1 | Hydrostatic Range 4 forward + 1 reverse, a mechanical combination of synchronizer and constant mesh spur gear with differential unit |  | Mechanical Combination of constant and Sliding mesh gears 03F+1R) |
| 4.0 | Technical Specifications: Drive System |  |  |
| 4.1 | Traction type: Wheel |  | Yes |
| 4.2 | Tyre size(s) |  |  |
| 4.2.1 | Front: | 16.9 X 30 (0.43 X 0.76m), 12 PR | 16.9-28 12PR |
| 4.2.2 | Rear: | 7.5 X 16 (0.19m X 0.41m), 12 PR | 7.50x16 16 PR |
| 4.4 | Ground clearance(mm): 360 |  | 380 |

| TECHNICAL SPECIFICATIONS REQUIRED |  | TECHNICAL SPECIFICATIONS OFFERED |
| --- | --- | --- |
|  |  | PLEASE FILL OUT ALL THE SPACES BELOW TO SPECIFY THE EQUIPMENT BEING OFFERED FOR THE OFFER TO BE VALID |
| 4.5 | Can move stably without bogging down on soft soils | Yes |
| 5.0 | Technical Specifications: Cutting table, reel and elevator |  |
| 5.1 | Effective Operating cutting width: 2 – 3 m | 3.6 m |
| 5.2 | Cutting height: Hydraulically operated | 0.90m , Yes |
| 5.3 | Pick-up reel: 4 sections, fitted with spring reel tines | 2 Sections Fitted With Spring Reel Tines |
| 5.4 | With declogging mechanism | No |
| 5.5 | Reciprocating cutting bar/knives | Yes |
| 5.6 | Reel Adjustment: Height and fore/aft adjustable | Yes |
| 5.7 | Reel Drive: Hydraulic control from the driver’s seat | Yes |
| 5.8 | Reel speed: Variable speed from 0 – 45 rpm | 48 RPM |
| 5.9 | Crop feed mechanism: Table auger, crop conveyor and feed roller | Yes |
| 6.0 | Technical Specifications: Threshing drum and concave |  |
| 6.1 | Concave is suitable for crops like rice and maize | Yes |
| 6.2 | Concave fitted with 8 bars | Yes |
| 6.3 | Threshing drum diameter: 620 mm | 600 mm |
| 6.4 | Drum width: 1650 mm | 895 mm |
| 6.5 | Drum fitted with 6 rasp bars | 8 |
| 6.6 | Drum speed: Stepless between 350 and 2000 rpm | 532 to 1013 |
| 6.7 | Drum fitted with speed indicator | Yes |
| 6.8 | Easy and complete removal of residual seeds remaining inside its equipment from the previous harvest | Yes |

| TECHNICAL SPECIFICATIONS REQUIRED |  | TECHNICAL SPECIFICATIONS OFFERED |
| --- | --- | --- |
|  |  | PLEASE FILL OUT ALL THE SPACES BELOW TO SPECIFY THE EQUIPMENT BEING OFFERED FOR THE OFFER TO BE VALID |
| 7.0 | Technical Specifications: Sieves |  |
| 7.1 | Cleaning area: Min. 0.65 m2 | 0.89 m2 |
| 7.2 | Sieve sizes: suitable for wheat, barley, maize and soybean | Yes |
| 7.3 | Fitted with a cleaning fan | Yes |
| 8.0 | Technical Specifications: Grain handling |  |
| 8.1 | Provide a feeding table for stationary threshing | No |
| 8.2 | Fitted with a side-mounted bagging device | No |
| 8.3 | Fitted with a pneumatic grain conveyor | No |
| 8.4 | Capable of effectively separating the grain from the straw or chaff without contamination. | Yes |
| 8.5 | Capable of effectively separating the grain from the straw or chaff without breaking the seeds. | Yes |
| 9.0 | Technical Specifications: Grain tank |  |
| 9.1 | Grain storage capacity: 2.0 - 3.0 m3 with hopper, two outlets | 1.7 m3 |
| 10.0 | Certification and Compliance with international standards |  |
| 10.1 | A copy of the latest and valid national or international standard certificate/ badge confirming the quality of the brand and model of the combine harvester delivered must be attached to your application. | - |
| 10.2 | ISO 4254-7:2017 and associated international standards | - |
| 10.3 | All moving and rotating shafts, pulleys, belts, and other hazardous parts to be covered and guarded | Yes |

| TECHNICAL SPECIFICATIONS REQUIRED |  | TECHNICAL SPECIFICATIONS OFFERED |
| --- | --- | --- |
|  |  | PLEASE FILL OUT ALL THE SPACES BELOW TO SPECIFY THE EQUIPMENT BEING OFFERED FOR THE OFFER TO BE VALID |
| 11.0 | Technical Specifications: Standard Tools and Accessories |  |
| 11.1 | Each unit is provided with a manufacturer’s set of standard tools for repair and maintenance. | Yes |
| 11.2 | First aid kit |  |
| 11.3 | Fire extinguisher |  |
| 11.4 | Each unit is provided with a manufacturer’s set of standard tools for repair and maintenance, a minimum of: |  |
| 11.4.1 | Three (3) pieces open wrenches |  |
| 11.4.2 | One (1) piece Philip or cross-tip screwdriver |  |
| 11.4.3 | One (1) piece flat screwdriver |  |
| 11.4.4 | One (1) piece of mechanical pliers |  |
| 11.4.5 | One (1) piece adjustable wrench |  |
| 11.4.6 | One (1) piece of grease gun |  |
| 11.4.7 | One (1) piece tire wrench |  |
| 11.4.8 | One (1) piece lifting jack |  |
| 11.4.9 | One (1) piece toolbox/carrier |  |
| 12.0 | Manuals: |  |
| 12.1 | Original Equipment Manufacturer's Manual (OEM) (written in English and printed or on CD and preferably in Persian languages) provided for each unit. Parts, Service, and User’s/Operators |  |
| 12.2 | Brochures, leaflets, quality certificates and/or manuals that provide information on the required technical specification of the offered item must be included in the documents in English and preferably in Persian languages, as well |  |
| 12.3 | The cost of the manuals should be included in the DAP/DAT cost of the equipment |  |
| 13.0 | Spare Parts: |  |

| TECHNICAL SPECIFICATIONS REQUIRED |  | TECHNICAL SPECIFICATIONS OFFERED |
| --- | --- | --- |
|  |  | PLEASE FILL OUT ALL THE SPACES BELOW TO SPECIFY THE EQUIPMENT BEING OFFERED FOR THE OFFER TO BE VALID |
| 13.1 | A brand with local spares backup supply easy to maintain and service |  |
| 13.2 | A list of the manufacturer’s recommended spare parts, sufficient for 2 years or a minimum of 2,000 hours of operation must be submitted for each unit. The list must be itemised, cost and submitted with your bid. |  |
| 14.0 | After-sales service: |  |
| 14.1 | Kindly indicate the name, Tel./Fax number and email or postal address of your service agents in Zimbabwe. |  |
| 14.2 | The bid price must include the cost of delivery and installation |  |
| 15.0 | Delivery, and Training |  |

| 15.1 | The bid price must include the cost of delivery, training, commissioning, and installation of the machine unit. |  |
| --- | --- | --- |
| 15.2 | Before delivery, the Technical Staff will conduct a bid post qualification inspection of the machine unit. Kindly, indicate who will be the person responsible and contact details. |  |
| 15.3 | The machine units must be delivered fully assembled and ready for use. |  |
| 15.4 | A final acceptance inspection will be conducted by Technical Staff. |  |
| 15.5 | Upon delivery, the supplier must provide training on the operation, maintenance, and troubleshooting of the machine unit. |  |
| 15.6 | Training Needs Description: Both theoretical and practical (handson) on how to operate, maintain, and troubleshoot the machine unit for 10 participants (approximately) |  |
|  | TECHNICAL SPECIFICATIONS REQUIRED | TECHNICAL SPECIFICATIONS OFFERED |
|  |  | PLEASE FILL OUT ALL THE SPACES BELOW TO SPECIFY THE EQUIPMENT BEING OFFERED FOR THE OFFER TO BE VALID |
| 16.0 | Warranty: |  |
| 16.1 | A full and clear manufacturer’s warranty statement transferable with the ownership of the equipment ( or end-user) must be submitted with your bid detailing the terms and conditions that are being offered. The minimum requirement is for a twelve (12) months warranty period. |  |
| 17.0 | Inspection: |  |
| 17.1 |  |  |