Technical Specifications for  Combine Harvester

| Description | Specifications | 987 |
| --- | --- | --- |
| Machine Type | Combine harvester with two headers for wheat and maize/corn. | Combine harvester with two headers for wheat and maize/corn. |
| Manufacturing Date | 2025-2026 | 2025-2026 |
| Engine Type | Diesel- water-cooled- 4 cylinders- Vertical | Diesel- water-cooled- 6 cylinders- Vertical |
| Speed | 40 km/h | 2.16 TO 30.25 km/h |
| Horse Power (hp) | 175 | 133 (hp) |
| Feeding Capacity (kg/s) | 8 | ….. |
| Power Rate (kw) | 129 | 98 AT 2300 RPM |
| Fuel Tank Capacity (ltr) | 300-350 | 365 (Ltr) |
| Transmission | Hydrostatic transmission | Mechanical combination of constant and Sliding mesh gears 03F+1R |
| Wheat Header (m) | 3-4 | 4.3 m |
| Maize Header (m) | 3.5-4.5 | 3.42 m |
| Threshing Drum Dia (mm) | 600 | 600 |
| Drum Speed Control | CTV Electric, hydraulic | mechanical |
| Separator Drum (L*D) | 2700*450 | N/A |
| Separator Drum Control | 4- gradual control | N/A |
| Cleaning Area (M2) | 2.8 | 2.33 (M2) |
| Grain Tank Capacity (Ltr) | 2800 | 2500 (Ltr) |
| Unloading Speed (Ltr/s) | 30 | 102 |
| Cutter Width (mtr) | 3.5-4 | 4.3 (mtr) |
| Straw Chopper | Double- rotor blades adjustable as needed ( no more than 2 cm chopping) | N/A |
| Rear Axle | Stable, extended, reinforced, Steel made, anti-rust, wide wheels, and hydraulic system | Stable, reinforced, Steel made, anti-rust, wide wheels, and hydraulic system |
| Total Dimensions |  |  |
| Total Length (mm) | 7500-8500 | 7050 |
| Total Width (mm) | 3000-3300 | 3020 |
| Total Height ( mm) | 3460-3680 | 3810 |
| Weight (kg) | 7000-7500 | 9580 Kg |
| Cabinet | Air-conditioned | Air-conditioned |
| Essential Requirements | Complete maintenance tools, spare parts catalogues, operation & maintenance guide (Arabic & English), jack lift, wheel brace, fire extinguisher |  |
| Warranty | No less than 3-year service warranty |  |